Dome operating procedure

- Power on scope, dome rotator, minicomputer (remote1), power up the laptop,

- RealVNC viewer to Remote1 on the laptop

- optional

Open the Arduino IDE on remote1 and check port etc.

Open serial monitor

Home the dome (G command)

Close the IDE

- Open theskyx

- Connect the mount (choose, settings, connect)

- Park then unpark the mount if needed

- Under tools select TCP server and make sure TPC is running (if not click on it)

- Open command prompt

- Cd C:\users\clark\documents\domebridge

- Type python theskyx_dome_bridge.py  (dome will rotate to north)

If you see an error saying there is another script running close and restart skyx.

Restart the bridge

- Select an object in skyx  (dome rotates to the calculated angle and actual tracking starts)

- Monitor the progress in the command window

Shutdown sequence

- In bridge shutdown procedure similar to shutdown when target > 10 deg

- In slyx park the mount

- Ctrl C to stop tracking and shut down bridge

6. Serial Commands Reference

All commands are entered in the Serial Monitor with "No Line Ending" selected.

| Command | Description | Range |
| --- | --- | --- |
| 0-359 | Move dome to specified angle | degrees |
| O | Open shutter | — |
| C | Close shutter | — |
| G | Go home (park) | — |
| S | Display status | — |
| AO# | Set shutter open time | 1–120 sec |
| AC# | Set shutter close time | 1–120 sec |
| AT# | Set tension relief time | 1–10 sec |
| P# | Set park offset | 0–359 deg |
| W# | Set slow-down PWM (min motor power) | 50–150 |
| D# | Set ramp speed (motor acceleration) | 1–20 ms |
| Q | Query all configuration settings | — |
| I | Interactive configuration menu | — |
| ? | Display help | — |

T-point model procedure

Create a horizon  -  display > & atmospheric option

Field of view  -  display > field of view indicator

Connect equipment  -  telescope, camera, & filter wheel

Home telescope

T-point should use no filter or luminous filter

Pick a point on the screen near the scope park position (about 53 deg up) and slew

Exposure to 5 sec

Exposure delay

Binning 3X3

Light

Auto dark

Subframe NOT checked

Automatically save photos

Simulate photo using DSS (for simulation only)

Take an image

Right click on image -  to image link

Select All Sky  -  (check Blind and the bottom options)

Click All Sky search button

- Click search a range of image scales (Pixel Scale = 206.265 × pixel_size / focal_length)

130mm telescope ASI 1600 camera 206.265X3.8um/910 = 8.6 (+ a little more)

11” RASA telescope ASI 6200 camera 206.265X3.76um/620 = 12.5 (+ a little more)

Telescope > Tpoint > Calibration Run > automated calibration

Setup > Exposure time = 5 sec

Image scale = (8.6 or 12.5 see above)

Binning = 3X3

Create pointing targets

Change exposure delay to 40 sec

Acquire pointing samples > start (bottom- start arrow)

After about 12 samples so supper model (bottom of TPoint settings window)

Supper model after about ½ of total samples hit apply button

Supper model after samples are done hit the apply button

Calibration run tab > finish button

Polar alignment tab > accurate polar alignment

Click through the screens and select the suggested star or shoes a star from the list.

Right click on the star and Slew to selected star from this screen

Camera tab and take a photo

Focus tools

Exposure time = 1 sec

Binning 3X3

Select take Photos Continuously

Automatically save photos

Return to polar alignment window > next button

Star will move > adjust mount to bring the star back to the crosshair

In the accurate polar alignment window hit continue > finish button

Abort taking pictures and return to take photo tab

Polar alignment is complete!

Test

Start bridge with home calibration = Y  OK

Start bridge with home calibration = N  OK

Start with shutter closed

Calibration = y OK

Tracking shouldn’t start until shutter is open?

Calibration = N

Start with shutter open

Shutdown sequence?

Calib	shutter open	shutter close

Y

n