Dome Controller V12.6
User Manual

Author: Clarke Yeager
Date: 4/3/2026

1. Overview

Dome Controller V12.6 is an Arduino-based observatory dome rotation controller. It drives a DC motor through an H-bridge to rotate the dome to specific azimuth positions (0–359 degrees), with position tracking via an encoder and external EEPROM storage.

V12.6 is the bridge-controlled version. All telescope tracking is handled by an external Python script (theskyx_dome_bridge.py) running on your PC, which polls TheSkyX for the telescope position and sends azimuth commands to the Arduino over a serial connection. The Arduino itself does not track – it only moves to the angle it receives.

2. Requirements

2.1 Hardware

•	Arduino board (must support interrupts on pins 9, 10, and 11)

•	H-bridge motor driver using 4 FETs (2 PWM on top, 2 digital on bottom)

•	DC motor with ring-and-pinion gear drive

•	Encoder/tachometer (24 pulses per motor revolution)

•	Two Hall effect sensors at home position (slow-down and stop)

•	External I2C EEPROM (24LC256 at address 0x57)

•	Three pushbuttons: CW, CCW, and Home

•	Shutter remote control with power LED indicator

2.2 Software

•	Arduino IDE

•	I2C_eeprom library by Rob Tillaart (install via Arduino Library Manager)

•	Python 3 with pyserial package (for the bridge script)

•	TheSkyX with TCP server enabled (port 3040)

3. Installation

3.1 Arduino Upload

1. Open Dome_Controller_V12.6.ino in the Arduino IDE.

2. Install the I2C_eeprom library if not already installed (Sketch > Include Library > Manage Libraries, search for "I2C_eeprom" by Rob Tillaart).

3. Select your board and COM port.

4. Upload.

3.2 Bridge Script Setup

1. Install Python 3 and the pyserial package:

pip install pyserial

2. Open theskyx_dome_bridge.py in a text editor.

3. Edit the configuration at the top of the file:

DOME_PORT = "COM3" – Change to your Arduino’s COM port

TSX_HOST = "localhost" – Change if TheSkyX runs on a different machine

TSX_PORT = 3040 – Default TheSkyX TCP port

4. In TheSkyX, enable the TCP server: Tools > TCP Server > check "Enable".

5. Run the bridge script:

python theskyx_dome_bridge.py

The bridge will print status messages as it monitors the telescope and commands the dome.

4. Serial Monitor Setup

IMPORTANT: When connecting to the Arduino via the Serial Monitor in the Arduino IDE, set the line ending dropdown to "No Line Ending". Baud rate: 9600.

On startup, the controller displays its version, date, and current configuration. It then prompts for an angle (0–359) or a command letter.

5. Operation

5.1 Manual Buttons

•	CW button – Press and hold to rotate the dome clockwise. The dome stops when you release the button.

•	CCW button – Press and hold to rotate the dome counter-clockwise. The dome stops when you release the button.

•	Home button – Press and hold for 2 seconds to initiate homing. The dome rotates to the north (0 degree) position using Hall effect sensors, then moves to the park offset position. Releasing before 2 seconds cancels the operation.

5.2 Moving to an Angle

Type an angle from 0 to 359 in the Serial Monitor and press Enter (or send from the bridge script). The controller:

1.	Reads the current position from memory.

2.	Calculates the shortest rotation path (CW or CCW) and the distance in degrees.

3.	If the distance is 10 degrees or more – CW move: Ramps to full speed, then automatically slows to slow speed within the final 10 degrees of travel to reduce coasting and stop close to the target.

4.	If the distance is 10 degrees or more – CCW move (backlash compensation): Rotates 10 degrees past the target CCW at full ramp speed. After the motor coasts to a stop, the actual encoder position is read to determine the exact overshoot distance. The dome then returns to the precise target position at slow speed in the CW direction. This eliminates gear backlash error and fully accounts for motor coast.

5.	If the distance is less than 10 degrees (short move): Rotates directly to the target at slow speed only, with no overshoot.

6.	Saves the new position to EEPROM.

5.3 Homing and Parking

The G command (or holding the Home button for 2 seconds) initiates homing:

1.	The dome rotates toward north using the shortest path.

2.	When the slow-down Hall sensor is triggered, the motor ramps down to minimum speed.

3.	The home stop Hall sensor triggers to pinpoint the north (0 degree) position.

4.	Position is reset to 0 in EEPROM.

5.	If a park offset is configured (P command), the dome moves to that position.

The park offset is an absolute dome position – telescope offset is not applied. This is used to position the dome opening for wind protection or preferred parking orientation.

Homing has a 60-second timeout per phase. If either Hall sensor is not detected within the timeout, homing fails with an error message.

5.4 Shutter Control

The shutter is controlled through a wireless remote. The Arduino holds the remote’s power switch until the power LED changes state, then activates the open or close command.

Open (O command):

1.	If the shutter is already OPEN, no action is taken.

2.	If the remote power LED is off, the controller holds the PW switch until the LED lights (polled every 50ms, 6-second timeout). If the LED does not light within 6 seconds, an error is reported.

3.	After a 100ms pause, the open signal is activated for the configured open time.

4.	During opening, you can cancel by pressing Y or any physical button.

5.	After opening, a brief tension relief pulse is applied (close direction, configurable duration) to reduce mechanical strain.

6.	The remote is powered off.

7.	Shutter state is saved to EEPROM as OPEN.

Close (C command):

Same sequence as Open but activates the close signal, and tension relief pulses in the open direction. Shutter state is saved as CLOSED.

Remote Power Error:

If the remote power LED does not respond within 6 seconds (either turning on or off), an error message is displayed: "ERR: Remote power failure. Check Remote battery." The shutter operation is aborted.

5.5 Bridge-Controlled Tracking

V12.6 does not have internal tracking. Instead, the Python bridge script (theskyx_dome_bridge.py) handles tracking by:

•	Polling TheSkyX every 5 seconds for the telescope’s azimuth, altitude, declination, and pier side.

•	Computing the correct dome azimuth using full 3D geometry (mount offsets, GEM arm, pier side) via calc_dome_azimuth().

•	Detecting slews: if the telescope azimuth changes by 10 degrees or more, the new dome angle is sent immediately.

•	Sending rotation updates whenever the calculated dome azimuth drifts 3 degrees from the last commanded position, keeping the dome continuously aligned during sidereal tracking.

•	Automatically parking the dome and closing the shutter when the target drops below 10 degrees altitude.

•	Handling meridian flips automatically – pier side is factored into the geometry calculation on every poll, no commands are sent to the Arduino.

To start tracking, simply run the bridge script while TheSkyX is connected to your telescope. To stop, close the bridge script (Ctrl+C).

6. Serial Commands Reference

All commands are entered in the Serial Monitor with "No Line Ending" selected.

| Command | Description | Range |
| --- | --- | --- |
| 0-359 | Move dome to specified angle | degrees |
| O | Open shutter | — |
| C | Close shutter | — |
| G | Go home (park) | — |
| S | Display status | — |
| AO# | Set shutter open time | 1–120 sec |
| AC# | Set shutter close time | 1–120 sec |
| AT# | Set tension relief time | 1–10 sec |
| P# | Set park offset | 0–359 deg |
| W# | Set slow-down PWM (min motor power) | 50–150 |
| D# | Set ramp speed (motor acceleration) | 1–20 ms |
| Q | Query all configuration settings | — |
| I | Interactive configuration menu | — |
| ? | Display help | — |

7. Dome Geometry (Bridge)

All dome azimuth correction is handled by the bridge script. The Arduino receives the final dome azimuth directly and moves to it – no offset calculation is performed on the Arduino.

The bridge computes the correct dome slit azimuth using full 3D geometry via calc_dome_azimuth(). This accounts for:

•	Mount RA axis position relative to dome center (MOUNT_DX, MOUNT_DY, MOUNT_DZ)

•	GEM declination arm swing with hour angle (DEC_ARM_LENGTH)

•	Pier side (east/west) – handled automatically on every poll, no commands sent to the Arduino

•	Observer latitude (LATITUDE) for accurate hour angle calculation

Meridian flips are handled automatically. Because pier side is factored into the geometry calculation on every 5-second poll, the dome azimuth corrects itself immediately after a flip with no manual intervention.

See Section 14 for the full list of geometry parameters and how to measure them.

8. Interactive Configuration Menu

Enter I to open the interactive configuration menu. This displays a numbered list of all configurable parameters with their current values:

1. Shutter open time (sec)

2. Shutter close time (sec)

3. Tension relief time (sec)

4. Park offset (deg)

5. Slow-down PWM

6. Ramp speed (ms)

Enter a number (1–6) to change a setting, then enter the new value when prompted. The value is validated against its allowed range.

Enter S to save all changes to EEPROM.

Enter X to exit. If you have unsaved changes, a warning is displayed and you must confirm with Y to exit without saving.

9. Configuration Defaults

| Parameter | Default | Range |
| --- | --- | --- |
| Shutter open time | 30 sec | 1–120 sec |
| Shutter close time | 30 sec | 1–120 sec |
| Tension relief time | 1 sec | 1–10 sec |
| Park offset | 140 deg | 0–359 deg |
| Slow-down PWM | 75 | 50–150 |
| Ramp speed | 3 ms | 1–20 ms |

Defaults are used when no valid configuration is found in EEPROM (first run or after EEPROM reset). All settings are saved to EEPROM and persist across power cycles.

10. Dome Geometry

The dome uses a ring-and-pinion gear system:

•	Dome ring: 175 holes

•	Drive gear: 14 cogs

•	Encoder: 24 pulses per motor revolution

•	Motor revolutions per dome revolution: 175 / 14 = 12.5

•	Encoder pulses per 360°: 24 × 12.5 = 300

•	Resolution: 360° / 300 = 1.2° per encoder pulse

The dome always takes the shortest rotation path to the target angle.

CW slow-down zone: For CW moves of 10 degrees or more, the motor runs at full speed then automatically switches to slow speed within the final 10 degrees of travel. This reduces coasting overshoot on CW moves.

Backlash compensation (CCW moves): For CCW moves of 10 degrees or more, the dome overshoots the target by 10 degrees at full speed. After the motor coasts to a stop, the controller reads the actual encoder position to calculate the exact return distance, then drives back to the target at slow speed in the CW direction. Using the actual post-coast position (rather than a fixed pulse count) ensures the dome lands precisely on the target regardless of how far the motor coasted. For moves of less than 10 degrees, the dome moves directly to the target at slow speed in either direction.

Encoder debounce: The encoder interrupt ignores any pulse that arrives within 50ms of the previous one. This filters electrical noise from the motor drive without affecting position accuracy (real pulses are at least 260ms apart at full speed).

11. Motor Control

The motor is driven through an H-bridge with 4 FET transistors:

•	Clockwise: Top Left FET (PWM on pin 3) + Bottom Right FET (digital on pin 2)

•	Counter-clockwise: Top Right FET (PWM on pin 5) + Bottom Left FET (digital on pin 4)

The motor ramps up from the slow-down PWM value to full power (255) at the configured ramp speed, and ramps down before stopping. This prevents sudden starts and stops that could jolt the dome.

Slow-down PWM (W command): Sets the minimum motor power level. Lower values allow slower, more precise movement but may stall on some motors. Range: 50–150.

Ramp speed (D command): Controls how quickly the motor accelerates and decelerates. Each step increases PWM by 1 and waits this many milliseconds. Lower values = faster ramp. Range: 1–20 ms.

A 30-second motor timeout protects against encoder failures. If no encoder pulses are detected for 30 seconds during a programmatic move, the motor stops with an error message.

12. EEPROM Storage

All configuration and the current dome position are stored in an external I2C EEPROM (24LC256). This preserves settings and position across power cycles.

On startup, the controller checks the EEPROM connection. If the EEPROM is not detected:

•	A warning is displayed.

•	Default configuration values are used.

•	Position is not saved (will be lost on power cycle).

•	The controller retries the EEPROM connection every 10 seconds and recovers automatically if reconnected.

A validity marker (0xAA) at EEPROM address 30 indicates whether saved configuration exists. On first run, defaults are used and the marker is written when you first save.

13. Error Messages

| Message | Meaning / Action |
| --- | --- |
| ERROR: EEPROM not responding | Check I2C wiring to 24LC256. Verify address 0x57. |
| WARNING: Invalid position read | EEPROM contains corrupt data. Home the dome to recalibrate. |
| ERROR: Motor timeout | No encoder pulses for 30 seconds. Check encoder and motor connections. |
| ERROR: Homing timeout | Hall sensor not detected within 60 seconds. Check Hall sensor wiring. |
| ERR: Remote power failure | Remote LED did not light within 6 seconds. Check remote battery. |
| ERR: Remote power off failed | Remote LED did not turn off within 6 seconds. Check remote battery. |

14. Bridge Script Configuration

The following parameters can be adjusted at the top of theskyx_dome_bridge.py:

| Parameter | Default | Description |
| --- | --- | --- |
| TSX_HOST | localhost | IP address of the TheSkyX machine |
| TSX_PORT | 3040 | TheSkyX TCP server port |
| DOME_PORT | COM3 | Serial port of the Arduino |
| POLL_INTERVAL | 5 | Seconds between telescope position checks |
| ROTATION_THRESHOLD | 3 | Dome azimuth drift (degrees) that triggers a tracking update |
| SLEW_THRESHOLD | 10 | Telescope azimuth change (degrees) that triggers an immediate slew response |
| HORIZON_ALTITUDE | 10 | Altitude (degrees) that triggers auto park and shutter close |
| PARK_CLOSE_DELAY | 60 | Seconds to wait for dome to reach park before closing shutter |
| AUTO_CLOSE_SHUTTER | True | Set to False to park only without closing shutter |
| DOME_RADIUS | 1.17 | Dome interior radius in meters |
| MOUNT_DX | 0.13 | Mount RA axis offset from dome center in meters (+ = East, - = West) |
| MOUNT_DY | 0.064 | Mount RA axis offset from dome center in meters (+ = North, - = South) |
| MOUNT_DZ | 0.33 | Mount RA axis height above dome center height in meters (+ = up) |
| LATITUDE | 40.1167 | Observer latitude in degrees North (40 deg 07 min) |
| DEC_ARM_LENGTH | 0.24 | Distance from RA axis to OTA mounting point in meters. Set 0 to disable GEM arm correction. |

The bridge computes the dome azimuth using full 3D geometry on every poll. Pier side is factored in automatically – no commands are sent to the Arduino for meridian flips. All azimuth correction is owned by the bridge; the Arduino simply moves to the angle it receives.

15. Typical Observing Session

1. Power on the Arduino. The controller displays its version and configuration.

2. Home the dome by holding the Home button for 2 seconds, or type G in the Serial Monitor. The dome finds north and moves to the park position.

3. Open the shutter by typing O. The controller powers on the remote, opens the shutter, applies tension relief, and powers off the remote.

4. Connect your telescope in TheSkyX and enable the TCP server.

5. Start the bridge script: python theskyx_dome_bridge.py

6. Slew your telescope to a target. The bridge detects the slew and sends the new azimuth to the dome.

7. During tracking, the bridge automatically sends a position update whenever the calculated dome azimuth drifts 3 degrees from the last commanded position, keeping the dome continuously aligned.

8. If the telescope crosses the meridian, the bridge handles the flip automatically. Pier side is factored into the dome geometry calculation on every 5-second poll – no manual action needed.

9. When the target altitude drops to HORIZON_ALTITUDE (10 degrees), the bridge automatically sends G to park the dome, waits PARK_CLOSE_DELAY (60 seconds), then sends C to close the shutter. No manual intervention is required.

10. When finished early, stop the bridge script (Ctrl+C), then manually close the shutter by typing C.

11. Park the dome by typing G.

16. Raspberry Pi Setup

The bridge script can run on a Raspberry Pi, allowing a small dedicated computer to manage dome tracking without tying up your main PC.

16.1 Requirements

•	Raspberry Pi (any model with USB and network connectivity)

•	Raspberry Pi OS installed (Python 3 is pre-installed)

•	USB cable from the Arduino to the Pi

•	Network connection to the PC running TheSkyX (same local network)

16.2 Install pyserial

Open a terminal on the Pi and run:

pip install pyserial

16.3 Copy the Bridge Script

Copy theskyx_dome_bridge.py to the Pi. You can use a USB flash drive, email, or transfer over the network. Place the file in your home folder (e.g., /home/pi/).

16.4 Edit the Script for Linux

Open the script in a text editor:

nano theskyx_dome_bridge.py

Serial port: On Linux, serial ports use a different naming convention than Windows. Plug the Arduino’s USB cable into the Pi, then find the port name:

ls /dev/ttyUSB* /dev/ttyACM*

You will see something like /dev/ttyUSB0 or /dev/ttyACM0. Change the DOME_PORT line in the script:

DOME_PORT = "/dev/ttyUSB0"

TheSkyX host: Since TheSkyX runs on your Windows PC (not on the Pi), change TSX_HOST to your PC’s IP address:

TSX_HOST = "192.168.1.100"

To find your PC’s IP address, open a command prompt on Windows and type ipconfig. Look for the IPv4 address under your network adapter. Both the Pi and the PC must be on the same network.

Save and exit nano by pressing Ctrl+X, then Y, then Enter.

16.5 Serial Port Permission

The Pi may block access to the serial port by default. Grant permission by adding your user to the dialout group:

sudo usermod -a -G dialout $USER

Reboot the Pi for the change to take effect:

sudo reboot

16.6 Run the Bridge Script (Recommended: Manual Start via SSH)

The recommended approach is to start the bridge manually when you are ready to observe. This ensures TheSkyX is running and the telescope is connected before the bridge tries to communicate with them.

From your Windows PC, open a command prompt or PowerShell and SSH into the Pi:

ssh pi@192.168.1.xxx

Replace 192.168.1.xxx with your Pi’s IP address. When prompted, enter your Pi’s password.

Then start the bridge script:

python3 theskyx_dome_bridge.py

The script will print status messages as it monitors the telescope and sends azimuth commands to the dome. Press Ctrl+C to stop the script when your observing session is finished.

16.7 Auto-Start on Boot (Not Recommended)

Auto-starting the bridge on boot is possible but not recommended because:

•	If TheSkyX is not running yet, the bridge will print repeated connection errors until it is started.

•	If the Arduino is not connected via USB, the bridge will crash with a serial port error.

•	You cannot use the Arduino Serial Monitor while the bridge holds the serial port.

If you still want auto-start, edit the crontab:

crontab -e

Add this line at the bottom of the file:

@reboot sleep 30 && python3 /home/pi/theskyx_dome_bridge.py >> /home/pi/dome_bridge.log 2>&1

This waits 30 seconds after boot (to allow the network to connect), then starts the bridge script. All output is saved to dome_bridge.log so you can review it later:

cat /home/pi/dome_bridge.log

To stop the auto-started bridge, find its process and stop it:

ps aux | grep dome_bridge

kill <process_id>

16.8 Important Notes

•	The Arduino Serial Monitor and the bridge script cannot both use the serial port at the same time. If the bridge is running on the Pi, do not also open the Serial Monitor on a PC connected to the same Arduino.

•	If the Pi loses network connectivity, the bridge script will print connection errors but will keep retrying automatically.

•	Make sure TheSkyX’s TCP server is enabled (Tools > TCP Server) and that your PC’s firewall allows incoming connections on port 3040.

17. Troubleshooting

Dome does not move:

•	Check that the encoder is connected to pin 11.

•	Verify the H-bridge FET connections.

•	Check the Serial Monitor for motor timeout errors.

Dome stops at wrong position:

•	Home the dome (G) to recalibrate the position.

•	Check the encoder for missed pulses.

•	Adjust the slow-down PWM (W) if the dome overshoots.

Dome offset is wrong after meridian flip:

•	Check that the bridge script is running and connected.

•	Pier side is handled automatically by the bridge geometry calculation. Verify the MOUNT_DX, MOUNT_DY, MOUNT_DZ, and DEC_ARM_LENGTH values are correct for your observatory.

Shutter does not open/close:

•	Check "ERR: Remote power failure" – replace remote battery.

•	Verify the power LED input on pin A0 is reading correctly.

•	Increase the open/close time (AO/AC) if the shutter does not fully open/close.

Homing fails:

•	Check Hall sensor wiring on pins 9 and 10.

•	Verify the slow-down and stop magnets are properly positioned.

•	Check for "Homing timeout" error messages in the Serial Monitor.

Bridge script errors:

•	"Connection refused" – Enable TheSkyX TCP server (Tools > TCP Server).

•	"Serial port not found" – Verify DOME_PORT matches your Arduino’s COM port.

•	"Access denied" on COM port – Close the Arduino Serial Monitor before running the bridge (only one program can use the COM port at a time).

18. Pin Reference

| Pin | Function | Type |
| --- | --- | --- |
| 2 | Bottom Right H-bridge FET | Digital output |
| 3 | Top Left H-bridge FET (CW drive) | PWM output |
| 4 | Bottom Left H-bridge FET | Digital output |
| 5 | Top Right H-bridge FET (CCW drive) | PWM output |
| 6 | CW manual button | Input (pullup) |
| 7 | CCW manual button | Input (pullup) |
| 8 | Home button | Input (pullup) |
| 9 | Home stop Hall sensor | Interrupt input |
| 10 | Home slow-down Hall sensor | Interrupt input |
| 11 | Encoder / tachometer | Interrupt input |
| A0 | Remote power LED | Analog input |
| A1 | Shutter open signal | Digital output |
| A2 | Remote PW switch | Digital output |
| A3 | Shutter close signal | Digital output |

Dome Controller V12.6 User Manual – Clarke Yeager – 4/3/2026