Running the TheSkyX Dome Bridge
on Windows

Author: Clarke Yeager
Date: 4/18/2026

1. Overview

theskyx_dome_bridge.py is a Python script that connects TheSkyX to the Dome Controller Arduino (V12.6). It runs in a Windows Command Prompt window and must be kept running for the entire observing session. It polls the telescope position every 5 seconds and sends azimuth commands to the Arduino to keep the dome slit aligned with the telescope.

This document covers one-time installation and configuration, plus the steps to follow each night.

2. One-Time Installation

2.1 Install Python 3

1.	Go to python.org and download the latest Python 3 installer.

2.	Run the installer. On the first screen, check "Add Python to PATH" before clicking Install.

3.	When installation is complete, open a Command Prompt and verify: python --version should print a version number.

2.2 Install pyserial

Open a Command Prompt (Start > type cmd > Enter) and run:

pip install pyserial

This is the only third-party package required.

2.3 Enable the TheSkyX TCP Server

1.	In TheSkyX, go to Tools > TCP Server.

2.	Check Enable. Leave the port at the default (3040) unless another program is already using it.

3.	Click OK. The TCP server will start automatically each time TheSkyX opens.

2.4 Find the Arduino COM Port

1.	Connect the Dome Controller Arduino to the PC via USB.

2.	Open Device Manager (Start > type devmgmt.msc > Enter).

3.	Expand Ports (COM & LPT). Note the COM port assigned to the Arduino (e.g., COM3).

4.	If you do not see an Arduino entry, install the Arduino USB driver from arduino.cc.

2.5 Configure the Bridge Script

Open theskyx_dome_bridge.py in Notepad or any text editor. Edit the constants at the top of the file:

| Setting | Default | What to change |
| --- | --- | --- |
| DOME_PORT | "COM3" | Set to your Arduino COM port |
| DOME_RADIUS | 1.17 | Dome interior radius in meters |
| MOUNT_DX | 0.13 | Mount RA axis east offset from dome center (meters, + = east) |
| MOUNT_DY | 0.064 | Mount RA axis north offset from dome center (meters, + = north) |
| MOUNT_DZ | 0.33 | Mount RA axis height above dome center height (meters, + = up) |
| LATITUDE | 40.1167 | Observer latitude in degrees North |
| DEC_ARM_LENGTH | 0.24 | GEM dec arm length from RA axis to OTA mounting point (meters) |
| AUTO_CLOSE_SHUTTER | True | Set False to skip auto shutter close on shutdown |
| TSX_PORT | 3040 | Change only if TheSkyX TCP port was changed |

Save the file after editing. You only need to do this once unless the hardware changes.

3. Starting the Bridge Each Night

Step 1 – Power on hardware

Power on the dome motor controller and the Arduino. Connect the Arduino USB cable to the PC if not already connected.

Step 2 – Start TheSkyX

Open TheSkyX and connect to the mount. The TCP server starts automatically if it was previously enabled. Verify the mount is connected and tracking before starting the bridge.

Step 3 – Open a Command Prompt

Press Win+R, type cmd, and press Enter. Navigate to the bridge folder:

cd "e:_Controller_V12.6"

Step 4 – Run the bridge

python theskyx_dome_bridge.py

Step 5 – Homing prompt

The bridge will ask:

Home dome for calibration? (Y/n):

Press Enter or Y to home the dome. The dome rotates to the home (north) sensor and then moves to the park offset. The bridge waits for the Arduino to confirm: "Park complete." This takes about 30-60 seconds depending on where the dome is starting from.

Press N only if the dome position is already calibrated from a recent session and you are certain the dome has not moved.

Important: Make sure TheSkyX is running and connected to the mount before pressing Y at the homing prompt. If TheSkyX is not ready, the bridge will fail to connect to the telescope after homing.

Step 6 – Shutter check

After homing, the bridge queries the Arduino for the current shutter state:

•	If the shutter is closed: the bridge opens it automatically. You will see "Opening shutter..." and then "Shutter opened."

•	If the shutter is already open: the bridge reports this and continues without issuing an open command.

•	If the shutter state is unknown: the bridge reports this and continues. Verify the shutter manually.

Step 7 – Tracking begins

The bridge connects to TheSkyX, takes the first telescope position, and enters the tracking loop. You will see:

Bridge running. Press Q to shut down for the night.

The bridge is now running. Slew the telescope to your first target in TheSkyX. The dome will follow.

4. During the Session

Leave the Command Prompt window open and visible. The bridge prints a status line each time it moves the dome or detects a significant event. Normal output looks like:

Rotation update: dome 142 -> 145 (tel az=140.2, alt=52.1)

New target (pier west): dome -> 210

Meridian flip detected (west -> east) - Flip confirmed - dome -> 168

You do not need to interact with the bridge during normal tracking. Meridian flips are handled automatically.

5. Shutting Down for the Night

Method 1 – Q key (recommended)

Click in the Command Prompt window and press Q. The bridge will ask:

Shutdown for the night? (Y/n):

Press Enter or Y. The bridge:

1.	Parks the dome (sends G, waits for "Park complete.")

2.	Closes the shutter (sends C, waits for "Shutter CLOSE sequence complete.")

3.	Prints "Shutdown complete." and exits

Wait for "Shutdown complete." before closing the window. Do not close the window before the shutter close is confirmed – closing the window resets the Arduino via USB and the shutter will not close.

Method 2 – Automatic (target below horizon)

If the telescope target drops below 10 degrees altitude, the bridge automatically parks the dome and closes the shutter, then exits. The same confirmation messages appear in the window.

6. Troubleshooting

"Error: another script is running"

A previous instance of the bridge left a script registered in TheSkyX. Fix:

1.	In TheSkyX, go to Script > Stop

2.	Run the bridge again

"Error: bad TheSkyX response"

TheSkyX returned a malformed reply. Usually clears on the next poll. If it continues, verify TheSkyX is connected to the mount and the telescope is not in an error state.

Homing timeout at startup

The Arduino reported "ERROR: Homing timeout". The dome did not reach the home sensor. Check:

•	Is the Arduino powered and connected?

•	Was TheSkyX running before you pressed Y at the homing prompt?

•	Is the home sensor aligned with the dome ring magnet?

Shutter does not close on shutdown

If the bridge prints "WARNING: Shutter close not confirmed":

•	The aperture remote may not have powered on – check the remote power LED

•	The shutter may have taken longer than 150 seconds – verify the AC (aperture close time) setting on the Arduino

•	Never close the Command Prompt window during shutdown – doing so resets the Arduino

Bridge fails to connect to COM port

The Arduino may not be powered on, or the USB cable is not seated. In Device Manager, verify the COM port is listed. Update the DOME_PORT value in the script if the port number has changed.

Dome moves to wrong position after restart

If you skipped homing at startup, the dome encoder position may be wrong. Stop the bridge (Ctrl+C), then restart and press Y to home. The dome will re-calibrate.

7. Arduino Serial Commands Reference

The bridge sends these commands automatically. You can also send them manually via the Arduino IDE Serial Monitor (set to No Line Ending, 9600 baud) while the bridge is not running.

| Command | Function |
| --- | --- |
| 0-359 | Move dome to that azimuth |
| G | Home dome and park at park offset |
| O | Open shutter |
| C | Close shutter |
| S | Display status (position, shutter state, settings) |
| Q | Query all EEPROM configuration settings |
| I | Interactive configuration menu |
| ? | Help (list all commands) |

8. Important Notes

•	Keep the Command Prompt window open for the entire session. Closing it resets the Arduino (USB DTR signal) and the shutter will not close automatically.

•	Do not run the Arduino Serial Monitor while the bridge is running. Only one program can hold the COM port at a time.

•	TheSkyX must be running before the bridge is started. If TheSkyX crashes and is restarted, you may need to restart the bridge as well (and possibly run Script > Stop in TheSkyX first).

•	Pier side is managed automatically by the bridge. It is factored into the dome geometry calculation on every poll – no manual commands are needed.

•	Always shut down using Q, not by closing the window or pressing Ctrl+C, to ensure the shutter closes properly.