TheSkyX Dome Bridge
User Manual

Author: Clarke Yeager
Date: 4/18/2026

1. Overview

theskyx_dome_bridge.py is a Python script that connects TheSkyX to the Dome Controller Arduino (V12.6). It polls the telescope position every 5 seconds, calculates the correct dome slit azimuth using full 3D observatory geometry, and sends the angle to the Arduino over a serial connection.

The bridge handles all dome tracking automatically:

•	Moves the dome immediately when a new slew is detected

•	Sends a tracking update whenever the calculated dome azimuth drifts 3 degrees from the last commanded position

•	Corrects for mount offset from dome center and GEM declination arm geometry

•	Detects meridian flips and updates pier side automatically

•	Parks the dome and closes the shutter when the target drops below the horizon

•	Provides a Q key for immediate end-of-night shutdown at any time

The Arduino receives only the final dome azimuth and moves to it. No offset calculation is performed on the Arduino.

2. Requirements

•	Python 3 (3.7 or later)

•	pyserial package

•	TheSkyX with TCP server enabled

•	Dome Controller Arduino (V12.6) connected via USB serial

•	Telescope connected and controlled by TheSkyX

3. Configuration

All configuration is at the top of theskyx_dome_bridge.py. Open the file in any text editor and edit the values before running.

3.1 Connection Settings

| Setting | Default | Description |
| --- | --- | --- |
| DOME_PORT | "COM3" | Serial port for Arduino USB connection |
| TSX_HOST | "localhost" | TheSkyX TCP server address |
| TSX_PORT | 3040 | TheSkyX TCP server port |

3.2 Observatory Geometry

| Setting | Default | Description |
| --- | --- | --- |
| DOME_RADIUS | 1.17 | Dome interior radius in meters |
| MOUNT_DX | 0.13 | Mount RA axis east offset from dome center (meters, + = east, - = west) |
| MOUNT_DY | 0.064 | Mount RA axis north offset from dome center (meters, + = north, - = south) |
| MOUNT_DZ | 0.33 | Mount RA axis height above dome center height (meters, + = up) |
| LATITUDE | 40.1167 | Observer latitude in degrees North |
| DEC_ARM_LENGTH | 0.24 | GEM declination arm length from RA axis to OTA mounting point (meters). Set 0 to disable. |

3.3 Behavior Settings

| Setting | Default | Description |
| --- | --- | --- |
| POLL_INTERVAL | 5 | Seconds between telescope position polls |
| ROTATION_THRESHOLD | 3 | Dome azimuth drift (degrees) that triggers a tracking update |
| SLEW_THRESHOLD | 10 | Telescope azimuth change (degrees) that triggers an immediate slew response |
| HORIZON_ALTITUDE | 10 | Altitude (degrees) that triggers auto park and shutter close |
| AUTO_CLOSE_SHUTTER | True | Close shutter automatically on park/shutdown |
| PARK_CLOSE_DELAY | 60 | Max seconds to wait for dome park confirmation |
| FLIP_MOTION_THRESHOLD | 1.0 | Scope azimuth change (deg/poll) that confirms a flip is in progress |

4. Starting the Bridge

Open a Command Prompt and run:

cd "e:_Controller_V12.6"
python theskyx_dome_bridge.py

4.1 Startup Sequence

When the bridge starts it performs the following steps in order:

1.	Connects to the Arduino on the configured COM port. If the port is not available the bridge exits immediately.

2.	Homing prompt – the bridge asks: Home dome for calibration? (Y/n): If you press Y (or Enter), the Arduino homes the dome and moves to the park offset. The bridge waits for the Arduino to report "Park complete." or "ERROR: Homing timeout" before continuing. If you press N, the dome is not moved and the current encoder position is used as-is. Skip homing only if the dome is already calibrated from a recent session.

3.	Connects to TheSkyX over TCP. If TheSkyX is not running or the TCP server is not enabled the bridge exits.

4.	Shutter state check – the bridge queries the Arduino for the current shutter (aperture) state. If the shutter is closed, it sends an O command to open it and waits for the open sequence to complete. If the shutter is already open it reports "Shutter already open" and continues. If the shutter state is unknown it reports this and continues without opening.

5.	First telescope poll – the initial azimuth and pier side are read from TheSkyX. The initial physical pier side is set from this first poll.

6.	Tracking loop starts – the bridge prints a reminder that Q can be pressed to shut down, then enters the 5-second polling loop.

5. During an Observing Session

5.1 Normal Tracking

Every 5 seconds the bridge queries TheSkyX for telescope azimuth, altitude, and pier side. If the computed dome azimuth has changed by more than SLEW_THRESHOLD (3 degrees) the Arduino is sent the new angle. The bridge reports the change:

Rotation update: dome 142 -> 145 (tel az=140.2, alt=52.1)

If the telescope is not moving (sidereal tracking only) the dome azimuth will drift slowly and periodic small corrections will be sent. This is normal.

5.2 New Target (Slew)

When TheSkyX reports scope motion above the slew threshold the bridge immediately repositions the dome:

New target (pier west): dome -> 210

The bridge waits for the slew to settle before resuming normal tracking.

5.3 Meridian Flip

A GEM mount performs a meridian flip when the telescope passes through the meridian. The bridge detects a flip when both of these are true:

•	The scope azimuth changes by more than FLIP_MOTION_THRESHOLD (1.0 deg/poll) and pier side has changed, and

•	The pier side reported by TheSkyX is different from the last confirmed pier side

When a flip is confirmed the bridge holds dome commands until the mount settles, then updates the physical pier side and moves the dome to the new position:

Flip detected - scope moving, pier side changed - holding dome...
Flip complete (pier east) - moving dome to 168 deg

Important: The bridge only updates the physical pier side from tracking motion if a confirmed slew was in progress (was_slewing flag). This prevents slow sidereal tracking drift from being misidentified as a flip. If you stop and restart TheSkyX tracking, the pier side will be re-confirmed on the next fast motion.

5.4 Target Below Minimum Altitude

When the telescope target drops below HORIZON_ALTITUDE (10 degrees) the bridge begins an automatic shutdown:

1.	Prints: Target below 10 deg - parking dome...

2.	Sends G (go home/park) to the Arduino

3.	Waits for "Park complete." from the Arduino (up to PARK_CLOSE_DELAY seconds). Reports: Dome parked (42 sec)

4.	If AUTO_CLOSE_SHUTTER is True, sends C to close the shutter and waits for "Shutter CLOSE sequence complete." (up to 150 seconds)

5.	Exits

If the Arduino reports "ERROR: Homing timeout", the bridge prints a warning, then still attempts to close the shutter before exiting. The shutter close command is sent regardless of whether homing succeeded, so the shutter will still close even if the dome did not reach the home sensor.

5.5 Q Key Shutdown

You can shut down the bridge at any time by pressing Q in the Command Prompt window where the bridge is running. The bridge will ask:

Shutdown for the night? (Y/n):

Press Enter or Y to confirm. The bridge then:

1.	Parks the dome (sends G, waits for "Park complete.")

2.	Closes the shutter if AUTO_CLOSE_SHUTTER is True (sends C, waits for "Shutter CLOSE sequence complete.")

3.	Prints Shutdown complete. and exits

Press N to cancel – the bridge continues normally.

Note: The serial port to the Arduino is kept open until the shutter close is confirmed. Closing the Command Prompt window or pressing Ctrl+C before shutdown completes will reset the Arduino (USB DTR reset) and the shutter will not close automatically.

6. Troubleshooting

6.1 "Error: another script is running"

TheSkyX only allows one JavaScript script at a time. If the bridge is restarted after a crash or forced quit, a previous script instance may still be registered in TheSkyX. To clear it:

1.	In TheSkyX, go to Script > Stop

2.	Restart the bridge

6.2 "Error: bad TheSkyX response"

The bridge received a malformed reply from TheSkyX. This usually means TheSkyX is busy or the TCP connection dropped momentarily. The bridge will retry on the next 5-second poll. If it continues, check that TheSkyX is connected to the mount and the telescope is tracking.

6.3 Homing Timeout at Startup

If the bridge reports "ERROR: Homing timeout" at startup, the dome did not reach the home sensor within the Arduino's timeout. Possible causes:

•	The Arduino was not connected or powered on

•	TheSkyX was not running when the homing prompt appeared (check this before answering Y)

•	The home sensor is not detecting the dome – verify sensor alignment

6.4 Shutter Not Closing on Shutdown

If the bridge reports "WARNING: Shutter close not confirmed", the Arduino did not print "Shutter CLOSE sequence complete." within 150 seconds. Possible causes:

•	The shutter motor did not run (check aperture remote power)

•	The shutter took longer than expected – verify the AC (aperture close time) setting on the Arduino

•	The Arduino was reset (USB DTR) – always let the bridge complete normally rather than force-closing the window

6.5 False Flip Detection

The bridge will not trigger a flip unless both conditions are met: the scope azimuth changed by more than 1.0 deg/poll and TheSkyX reports a different pier side. Slow tracking drift accumulating over many polls cannot trigger a flip because the bridge tracks whether a confirmed fast slew was in progress (was_slewing).

6.6 Dome Moves Unexpectedly After Restarting Bridge

If you skipped homing at startup and the dome encoder position is wrong, the dome may jump to an incorrect position on the first poll. Always home the dome at the start of each night unless you are certain the dome position has not changed since the last session.

7. Serial Commands Sent by the Bridge

The bridge sends these commands to the Arduino over the serial connection. You do not normally need to send these manually.

| Command | Purpose |
| --- | --- |
| 0-359 | Move dome to that azimuth |
| G | Go home and park dome |
| O | Open shutter |
| C | Close shutter |
| S | Query Arduino status (used for shutter state check) |

8. Typical Session Workflow

1.	Power on the Arduino and dome motor controller.

2.	Start TheSkyX and connect to the mount.

3.	Open a Command Prompt and run python theskyx_dome_bridge.py.

4.	At the homing prompt, press Enter (or Y) to home the dome. Wait for "Park complete."

5.	The bridge checks the shutter state and opens it if needed.

6.	The bridge begins polling. Slew the telescope to your first target in TheSkyX. The dome follows automatically.

7.	Observe. The bridge updates the dome as the target drifts. Meridian flips are handled automatically.

8.	When finished, press Q in the bridge window and confirm with Enter. The dome parks and the shutter closes.

9.	Wait for "Shutdown complete." before closing the Command Prompt window.

10.	Power off the mount and Arduino.

9. Restarting Bridge Tracking

If you need to stop and restart the bridge mid-session (for example after an error):

1.	Press Ctrl+C in the bridge window to stop it.

2.	If TheSkyX shows "another script is running", go to Script > Stop in TheSkyX.

3.	Run python theskyx_dome_bridge.py again.

4.	At the homing prompt, press N to skip homing if the dome position is known to be correct.

5.	The bridge reconnects and resumes tracking from the current position.

10. Performing a Meridian Flip

A meridian flip is initiated in TheSkyX, not in the bridge. The bridge detects and handles it automatically:

1.	In TheSkyX, command the mount to flip (the mount rotates from one side of the meridian to the other).

2.	While the mount is slewing, the bridge sees a large azimuth change and a pier side change in the same poll.

3.	The bridge confirms the flip, updates the physical pier side, and moves the dome to the new position.

4.	You do not need to stop or restart the bridge for a flip.

5.	If you manually flip outside of TheSkyX tracking, restart the bridge so the pier side state is re-confirmed from the first poll.